FAQ 五：The specific operation method of pu foam during use.

Gun type：

- Take a polyurethane foam gun and clean it with cleaner to ensure smoothly; then take a bottle of pu foam, hold the middle of the can tightly by hand, and shake it at least 20 times along the lateral direction of the can. make sure the prepolymer in the can is fully mixed;

- 2) Remove the protective cover of the valve on the pu foam coil, remove the cleaner on the foam gun, and put the pu foam can onto the interface of the foam gun along the thread;

3) Hold the foam gun handle and ensure that the can is placed on top of it. The foam gun is below the can and open the flow control valve at the back of the foam gun.

4) The tip of the gun is aimed at the bottom or inside of the part to be filled, and the trigger is gently pulled to start the construction. It can be adjusted to the appropriate flow through the flow adjustment valve behind the foam gun. After a can is sprayed, if it still needs to be filled, the empty cans can be removed immediately, and the foaming agent can be re-installed according to the method of point 3, and the construction should be continued. The interval time should not be too long.

5) After the construction is completed, remove the can in time and replace it with a special cleaner to clean the foam gun so as to prevent the residual material from solidifying and clogging the foaming gun.

6) Straw type product Reference the use of gun-type products, the use does not need to use a foam gun, just use a straw, convenient and quick。